[NW->MS] MIB (DL_FREQ_BAND[78], SSB_ARFCN[620352], PCI[0], SCS[NR_SCS_30])

Message Content

NR Radio Resource Control (RRC) protocol

BCCH-BCH-Message

message: mib (0)

mib

systemFrameNumber: 7c [bit length 6, 2 LSB pad bits, 0111 11.. decimal value 31]

subCarrierSpacingCommon: scs30or120 (1)

ssb-SubcarrierOffset: 6

dmrs-TypeA-Position: pos2 (0)

pdcch-ConfigSIB1

controlResourceSetZero: 2

searchSpaceZero: 0

cellBarred: notBarred (1)

intraFreqReselection: allowed (0)

spare: 00 [bit length 1, 7 LSB pad bits, 0... .... decimal value 0]

[NW->MS] SIB1 (DL_FREQ_BAND[78], SSB_ARFCN[[620352], PCI[0], SCS[NR_SCS_30])

Message Content

NR Radio Resource Control (RRC) protocol

BCCH-DL-SCH-Message

message: c1 (0)

c1: systemInformationBlockType1 (1)

systemInformationBlockType1

cellSelectionInfo

q-RxLevMin: -140dBm (-70)

q-RxLevMinSUL: -140dBm (-70)

q-QualMin: -20 dB

cellAccessRelatedInfo

plmn-IdentityInfoList: 1 item

Item 0

PLMN-IdentityInfo

plmn-IdentityList: 1 item

Item 0

PLMN-Identity

mcc: 3 items

Item 0

MCC-MNC-Digit: 0

Item 1

MCC-MNC-Digit: 0

Item 2

MCC-MNC-Digit: 1

mnc: 2 items

Item 0

MCC-MNC-Digit: 0

Item 1

MCC-MNC-Digit: 1

trackingAreaCode: 000001 [bit length 24, 0000 0000  0000 0000  0000 0001 decimal value 1]

ranac: 0

cellIdentity: 0000004000 [bit length 36, 4 LSB pad bits, 0000 0000  0000 0000  0000 0000  0100 0000  0000 .... decimal value 1024]

cellReservedForOperatorUse: notReserved (1)

connEstFailureControl

connEstFailCount: n1 (0)

connEstFailOffsetValidity: s30 (0)

connEstFailOffset: 1 dB

servingCellConfigCommon

downlinkConfigCommon

frequencyInfoDL

frequencyBandList: 1 item

Item 0

NR-MultiBandInfo

freqBandIndicatorNR: 78

offsetToPointA: 4 PRBs

scs-SpecificCarrierList: 1 item

Item 0

SCS-SpecificCarrier

offsetToCarrier: 0

subcarrierSpacing: kHz30 (1)

carrierBandwidth: 273

initialDownlinkBWP

genericParameters

locationAndBandwidth: 1099

subcarrierSpacing: kHz30 (1)

pdcch-ConfigCommon: setup (1)

setup

controlResourceSetZero: 2

searchSpaceZero: 0

commonSearchSpaceList: 1 item

Item 0

SearchSpace

searchSpaceId: 1

controlResourceSetId: 0

monitoringSlotPeriodicityAndOffset: sl1 (0)

sl1: NULL

monitoringSymbolsWithinSlot: 8000 [bit length 14, 2 LSB pad bits, 1000 0000  0000 00.. decimal value 8192]

nrofCandidates

aggregationLevel1: n0 (0)

aggregationLevel2: n2 (2)

aggregationLevel4: n1 (1)

aggregationLevel8: n0 (0)

aggregationLevel16: n0 (0)

searchSpaceType: common (0)

common

dci-Format0-0-AndFormat1-0

searchSpaceSIB1: 0

searchSpaceOtherSystemInformation: 0

pagingSearchSpace: 0

ra-SearchSpace: 1

pdsch-ConfigCommon: setup (1)

setup

pdsch-TimeDomainAllocationList: 5 items

Item 0

PDSCH-TimeDomainResourceAllocation

mappingType: typeA (0)

startSymbolAndLength: 67

Item 1

PDSCH-TimeDomainResourceAllocation

mappingType: typeA (0)

startSymbolAndLength: 82

Item 2

PDSCH-TimeDomainResourceAllocation

mappingType: typeA (0)

startSymbolAndLength: 53

Item 3

PDSCH-TimeDomainResourceAllocation

mappingType: typeA (0)

startSymbolAndLength: 72

Item 4

PDSCH-TimeDomainResourceAllocation

mappingType: typeA (0)

startSymbolAndLength: 44

bcch-Config

modificationPeriodCoeff: n4 (1)

pcch-Config

defaultPagingCycle: rf32 (0)

nAndPagingFrameOffset: halfT (1)

halfT: 0

ns: one (2)

uplinkConfigCommon

frequencyInfoUL

scs-SpecificCarrierList: 1 item

Item 0

SCS-SpecificCarrier

offsetToCarrier: 0

subcarrierSpacing: kHz30 (1)

carrierBandwidth: 273

p-Max: 23 dBm

initialUplinkBWP

genericParameters

locationAndBandwidth: 1099

subcarrierSpacing: kHz30 (1)

rach-ConfigCommon: setup (1)

setup

rach-ConfigGeneric

prach-ConfigurationIndex: 79

msg1-FDM: four (2)

msg1-FrequencyStart: 0

zeroCorrelationZoneConfig: 15

preambleReceivedTargetPower: -92 dBm

preambleTransMax: n100 (9)

powerRampingStep: dB4 (2)

ra-ResponseWindow: sl20 (5)

ssb-perRACH-OccasionAndCB-PreamblesPerSSB: one (3)

one: n8 (1)

ra-ContentionResolutionTimer: sf64 (7)

rsrp-ThresholdSSB: SS-RSRP < -156dBm (0)

prach-RootSequenceIndex: l139 (1)

l139: 0

msg1-SubcarrierSpacing: kHz30 (1)

restrictedSetConfig: unrestrictedSet (0)

pusch-ConfigCommon: setup (1)

setup

pusch-TimeDomainAllocationList: 3 items

Item 0

PUSCH-TimeDomainResourceAllocation

k2: 4

mappingType: typeA (0)

startSymbolAndLength: 27

Item 1

PUSCH-TimeDomainResourceAllocation

k2: 6

mappingType: typeA (0)

startSymbolAndLength: 27

Item 2

PUSCH-TimeDomainResourceAllocation

k2: 2

mappingType: typeA (0)

startSymbolAndLength: 27

msg3-DeltaPreamble: 2dB (1)

p0-NominalWithGrant: -90 dBm

pucch-ConfigCommon: setup (1)

setup

pucch-ResourceCommon: 0

pucch-GroupHopping: enable (1)

p0-nominal: -90 dBm

timeAlignmentTimerCommon: infinity (7)

ssb-PositionsInBurst

inOneGroup: 40 [bit length 8, 0100 0000 decimal value 64]

ssb-PeriodicityServingCell: ms20 (2)

tdd-UL-DL-ConfigurationCommon

referenceSubcarrierSpacing: kHz30 (1)

pattern1

dl-UL-TransmissionPeriodicity: ms5 (6)

nrofDownlinkSlots: 8

nrofDownlinkSymbols: 10

nrofUplinkSlots: 1

nrofUplinkSymbols: 2

ss-PBCH-BlockPower: 0 dBm

ue-TimersAndConstants

t300: ms2000 (7)

t301: ms1000 (5)

t310: ms1000 (5)

n310: n1 (0)

t311: ms30000 (6)

n311: n1 (0)

t319: ms1000 (5)

1-   [MS->NW] 5GMM_REGISTRATION_REQUEST (registration type: INITIAL_REGISTRATION, FOR bit: 0, ng_ran_rcu: 0, NAS message container included: KAL_TRUE)

Message Content

Non-Access-Stratum 5GS (NAS)PDUs

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NAS message container

Element ID: 0x71

Length: 40

Non-Access-Stratum 5GS (NAS)PDU

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

5GMM capability

Element ID: 0x10

Length: 3

0... .... = Service gap control (SGC): Not supported

.0.. .... = IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

...0 .... = Control plane CIoT 5GS optimization (5G-CP CIoT): Not supported

.... 0... = Restriction on use of enhanced coverage support (RestrictEC): Not supported

.... .1.. = LTE Positioning Protocol (LPP) capability: Supported

.... ..1. = ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach): Supported

.... ...0 = EPC NAS supported (S1 mode): Not supported

0... .... = Radio capability signalling optimisation (RACS) capability: Not supported

.1.. .... = Network slice-specific authentication and authorization (NSSAA): Supported

..1. .... = Location Services (5G-LCS) notification mechanisms capability: Supported

...0 .... = V2X communication over NR-PC5 capability (V2XCNPC5): Not supported

.... 0... = V2X communication over E-UTRA-PC5 capability (V2XCEPC5): Not supported

.... .0.. = V2X capability (V2X): Not supported

.... ..0. = User plane CIoT 5GS optimization (5G-UP CIoT): Not supported

.... ...0 = 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability: Not supported

0... .... = Spare: 0

.0.. .... = Spare: 0

..0. .... = Spare: 0

...0 .... = Spare: 0

.... 0... = Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT): Not supported

.... .0.. = Multiple user-plane resources support (multipleUP): Not supported

.... ..0. = WUS assistance (WUSA) information reception capability: Not supported

.... ...0 = Closed Access Group (CAG) capability: Not supported

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NSSAI - Requested NSSAI

Element ID: 0x2f

Length: 2

S-NSSAI 1

Length: 1

Slice/service type (SST): eMBB (1)

5GS tracking area identity - Last visited registered TAI

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

5GS update type

Element ID: 0x53

Length: 1

0... .... = Spare: 0

.0.. .... = Spare: 0

..00 .... = EPS Preferred CIoT network behaviour (EPS-PNB-CIoT): no additional information (0)

.... 00.. = 5GS Preferred CIoT network behaviour (5GS PNB-CIoT): no additional information (0)

.... ..0. = NG-RAN Radio Capability Update (NG-RAN-RCU): Not Needed

.... ...1 = SMS over NAS transport requested (SMS requested): Supported

Non-Access-Stratum 5GS (NAS)PDU

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NAS message container

Element ID: 0x71

Length: 40

Non-Access-Stratum 5GS (NAS)PDU

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

5GMM capability

Element ID: 0x10

Length: 3

0... .... = Service gap control (SGC): Not supported

.0.. .... = IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

...0 .... = Control plane CIoT 5GS optimization (5G-CP CIoT): Not supported

.... 0... = Restriction on use of enhanced coverage support (RestrictEC): Not supported

.... .1.. = LTE Positioning Protocol (LPP) capability: Supported

.... ..1. = ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach): Supported

.... ...0 = EPC NAS supported (S1 mode): Not supported

0... .... = Radio capability signalling optimisation (RACS) capability: Not supported

.1.. .... = Network slice-specific authentication and authorization (NSSAA): Supported

..1. .... = Location Services (5G-LCS) notification mechanisms capability: Supported

...0 .... = V2X communication over NR-PC5 capability (V2XCNPC5): Not supported

.... 0... = V2X communication over E-UTRA-PC5 capability (V2XCEPC5): Not supported

.... .0.. = V2X capability (V2X): Not supported

.... ..0. = User plane CIoT 5GS optimization (5G-UP CIoT): Not supported

.... ...0 = 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability: Not supported

0... .... = Spare: 0

.0.. .... = Spare: 0

..0. .... = Spare: 0

...0 .... = Spare: 0

.... 0... = Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT): Not supported

.... .0.. = Multiple user-plane resources support (multipleUP): Not supported

.... ..0. = WUS assistance (WUSA) information reception capability: Not supported

.... ...0 = Closed Access Group (CAG) capability: Not supported

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NSSAI - Requested NSSAI

Element ID: 0x2f

Length: 2

S-NSSAI 1

Length: 1

Slice/service type (SST): eMBB (1)

5GS tracking area identity - Last visited registered TAI

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

5GS update type

Element ID: 0x53

Length: 1

0... .... = Spare: 0

.0.. .... = Spare: 0

..00 .... = EPS Preferred CIoT network behaviour (EPS-PNB-CIoT): no additional information (0)

.... 00.. = 5GS Preferred CIoT network behaviour (5GS PNB-CIoT): no additional information (0)

.... ..0. = NG-RAN Radio Capability Update (NG-RAN-RCU): Not Needed

.... ...1 = SMS over NAS transport requested (SMS requested): Supported

Extended protocol discriminator: 5G mobility management messages (126)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

Length: 11

1... .... = Spare: 1

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

Non-Access-Stratum 5GS (NAS)PDU

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

5GMM capability

Element ID: 0x10

Length: 3

0... .... = Service gap control (SGC): Not supported

.0.. .... = IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

...0 .... = Control plane CIoT 5GS optimization (5G-CP CIoT): Not supported

.... 0... = Restriction on use of enhanced coverage support (RestrictEC): Not supported

.... .1.. = LTE Positioning Protocol (LPP) capability: Supported

.... ..1. = ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach): Supported

.... ...0 = EPC NAS supported (S1 mode): Not supported

0... .... = Radio capability signalling optimisation (RACS) capability: Not supported

.1.. .... = Network slice-specific authentication and authorization (NSSAA): Supported

..1. .... = Location Services (5G-LCS) notification mechanisms capability: Supported

...0 .... = V2X communication over NR-PC5 capability (V2XCNPC5): Not supported

.... 0... = V2X communication over E-UTRA-PC5 capability (V2XCEPC5): Not supported

.... .0.. = V2X capability (V2X): Not supported

.... ..0. = User plane CIoT 5GS optimization (5G-UP CIoT): Not supported

.... ...0 = 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability: Not supported

0... .... = Spare: 0

.0.. .... = Spare: 0

..0. .... = Spare: 0

...0 .... = Spare: 0

.... 0... = Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT): Not supported

.... .0.. = Multiple user-plane resources support (multipleUP): Not supported

.... ..0. = WUS assistance (WUSA) information reception capability: Not supported

.... ...0 = Closed Access Group (CAG) capability: Not supported

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NSSAI - Requested NSSAI

Element ID: 0x2f

Length: 2

S-NSSAI 1

Length: 1

Slice/service type (SST): eMBB (1)

5GS tracking area identity - Last visited registered TAI

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

5GS update type

Element ID: 0x53

Length: 1

0... .... = Spare: 0

.0.. .... = Spare: 0

..00 .... = EPS Preferred CIoT network behaviour (EPS-PNB-CIoT): no additional information (0)

.... 00.. = 5GS Preferred CIoT network behaviour (5GS PNB-CIoT): no additional information (0)

.... ..0. = NG-RAN Radio Capability Update (NG-RAN-RCU): Not Needed

.... ...1 = SMS over NAS transport requested (SMS requested): Supported

Plain NAS 5GS Message

Extended protocol discriminator: 5G mobility management messages (126)

0000 .... = Spare Half Octet: 0

.... 0000 = Security header type: Plain NAS message, not security protected (0)

Message type: Registration request (0x41)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

NAS key set identifier

0... .... = Type of security context flag (TSC): Native security context (for KSIAMF)

.000 .... = NAS key set identifier: 0

5GS mobile identity

Length: 11

1... .... = Spare: 1

.1.. .... = Spare: 1

..1. .... = Spare: 1

...1 .... = Spare: 1

.... 0... = Spare: 0

.... .010 = Type of identity: 5G-GUTI (2)

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

AMF Region ID: 254

0000 0000 01.. .... = AMF Set ID: 1

..00 0001 = AMF Pointer: 1

5G-TMSI: 1 (0x00000001)

5GMM capability

Element ID: 0x10

Length: 3

0... .... = Service gap control (SGC): Not supported

.0.. .... = IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

...0 .... = Control plane CIoT 5GS optimization (5G-CP CIoT): Not supported

.... 0... = Restriction on use of enhanced coverage support (RestrictEC): Not supported

.... .1.. = LTE Positioning Protocol (LPP) capability: Supported

.... ..1. = ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach): Supported

.... ...0 = EPC NAS supported (S1 mode): Not supported

0... .... = Radio capability signalling optimisation (RACS) capability: Not supported

.1.. .... = Network slice-specific authentication and authorization (NSSAA): Supported

..1. .... = Location Services (5G-LCS) notification mechanisms capability: Supported

...0 .... = V2X communication over NR-PC5 capability (V2XCNPC5): Not supported

.... 0... = V2X communication over E-UTRA-PC5 capability (V2XCEPC5): Not supported

.... .0.. = V2X capability (V2X): Not supported

.... ..0. = User plane CIoT 5GS optimization (5G-UP CIoT): Not supported

.... ...0 = 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability: Not supported

0... .... = Spare: 0

.0.. .... = Spare: 0

..0. .... = Spare: 0

...0 .... = Spare: 0

.... 0... = Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT): Not supported

.... .0.. = Multiple user-plane resources support (multipleUP): Not supported

.... ..0. = WUS assistance (WUSA) information reception capability: Not supported

.... ...0 = Closed Access Group (CAG) capability: Not supported

UE security capability

Element ID: 0x2e

Length: 2

1... .... = 5G-EA0: Supported

.1.. .... = 128-5G-EA1: Supported

..1. .... = 128-5G-EA2: Supported

...1 .... = 128-5G-EA3: Supported

.... 0... = 5G-EA4: Not supported

.... .0.. = 5G-EA5: Not supported

.... ..0. = 5G-EA6: Not supported

.... ...0 = 5G-EA7: Not supported

1... .... = 5G-IA0: Supported

.1.. .... = 128-5G-IA1: Supported

..1. .... = 128-5G-IA2: Supported

...1 .... = 128-5G-IA3: Supported

.... 0... = 5G-IA4: Not supported

.... .0.. = 5G-IA5: Not supported

.... ..0. = 5G-IA6: Not supported

.... ...0 = 5G-IA7: Not supported

NSSAI - Requested NSSAI

Element ID: 0x2f

Length: 2

S-NSSAI 1

Length: 1

Slice/service type (SST): eMBB (1)

5GS tracking area identity - Last visited registered TAI

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

5GS update type

Element ID: 0x53

Length: 1

0... .... = Spare: 0

.0.. .... = Spare: 0

..00 .... = EPS Preferred CIoT network behaviour (EPS-PNB-CIoT): no additional information (0)

.... 00.. = 5GS Preferred CIoT network behaviour (5GS PNB-CIoT): no additional information (0)

.... ..0. = NG-RAN Radio Capability Update (NG-RAN-RCU): Not Needed

.... ...1 = SMS over NAS transport requested (SMS requested): Supported

Extended protocol discriminator: 5G mobility management messages (126)

5GS registration type

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

.... .001 = 5GS registration type: initial registration (1)

.... 0... = Follow-On Request bit (FOR): No follow-on request pending

5GMM capability

Element ID: 0x10

Length: 3

0... .... = Service gap control (SGC): Not supported

.0.. .... = IP header compression for control plane CIoT 5GS optimization (5G-IPHC-CP CIoT): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

...0 .... = Control plane CIoT 5GS optimization (5G-CP CIoT): Not supported

.... 0... = Restriction on use of enhanced coverage support (RestrictEC): Not supported

.... .1.. = LTE Positioning Protocol (LPP) capability: Supported

.... ..1. = ATTACH REQUEST message containing PDN CONNECTIVITY REQUEST message for handover support (HO attach): Supported

.... ...0 = EPC NAS supported (S1 mode): Not supported

0... .... = Radio capability signalling optimisation (RACS) capability: Not supported

.1.. .... = Network slice-specific authentication and authorization (NSSAA): Supported

..1. .... = Location Services (5G-LCS) notification mechanisms capability: Supported

...0 .... = V2X communication over NR-PC5 capability (V2XCNPC5): Not supported

.... 0... = V2X communication over E-UTRA-PC5 capability (V2XCEPC5): Not supported

.... .0.. = V2X capability (V2X): Not supported

.... ..0. = User plane CIoT 5GS optimization (5G-UP CIoT): Not supported

.... ...0 = 5G-SRVCC from NG-RAN to UTRAN (5GSRVCC) capability: Not supported

0... .... = Spare: 0

.0.. .... = Spare: 0

..0. .... = Spare: 0

...0 .... = Spare: 0

.... 0... = Ethernet header compression for control plane CIoT 5GS optimization (5G-EHC-CP CIoT): Not supported

.... .0.. = Multiple user-plane resources support (multipleUP): Not supported

.... ..0. = WUS assistance (WUSA) information reception capability: Not supported

.... ...0 = Closed Access Group (CAG) capability: Not supported

Element ID: 0x10

0... .... = Service gap control (SGC): Not supported

..0. .... = N3 data transfer (N3 data): Not supported

5GS tracking area identity - Last visited registered TAI

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

Element ID: 0x52

Mobile Country Code (MCC): Unknown (1)

Mobile Network Code (MNC): Unknown (01)

TAC: 1

..00 .... = EPS Preferred CIoT network behaviour (EPS-PNB-CIoT): no additional information (0)